Solar Declination Coordinate Correction

Investigation, Finding, and Resolution

Planetary Sunshade Institute — Science Team Reference Document

| Document type | Internal science reference — PSF MATLAB model and CESM interface |
| --- | --- |
| Status | fix implemented and verified (April 2026) |
| Affects | All preconfigured-mode .NC output files; Architecture B in particular |
| Files changed | analysis_General___0___X.m ¦ spencer_declination.m (new) |

Executive Summary

Key finding The PSF MATLAB model computed irradiance factors (IF) in a heliocentric frame where phi = 0 is always the sub-stellar point, but then wrote these values to the.NC file as if phi were geographic latitude. The result: the shadow footprint in every .NC file appeared permanently centred at the geographic equator, regardless of the time of year. The fix applies the solar declination angle δ(day) — computed analytically from the Spencer (1971) formula — to shift the heliocentric IF column into geographic latitude space before writing to the .NC file.

CESM handles Earth’s axial tilt internally: its orbital library computes the solar declination angle at every timestep, and this feeds directly into the zenith-angle calculation that projects top-of-atmosphere flux onto each grid column. The .NC input file does not need to encode the seasonal migration of the sub-stellar point — that geometry is CESM’s responsibility.

What the sun_shade file must encode is the geographic latitude at which each fraction of irradiance is blocked. This is a function of where the constellation shadow footprint actually falls on Earth’s surface for a given day of the year. The heliocentric computation frame is correct for the physics calculation, but its results must be rotated by δ(day) before they are meaningful as geographic quantities.

The impact was relatively small but still noteworthy for Architecture A, (shade center is close to equator year-round) but significant for Architecture B, where off-equatorial shading elements are designed to move relative to the sub-stellar point. For Architecture B, the un-corrected file would have systematically placed polar-latitude shading closer to the equator (and produced findings that shade craft above/below the sun-Earth line are less effective than they actually are).

Part I — CESM Solar Geometry

## 1.1  The CESM Radiation Pipeline

## CESM computes the top-of-atmosphere insolation for every atmospheric column at every timestep via a well-established pipeline. The core calculation in the RRTMG shortwave interface is:

## solin(i) = ETF × sun_shade(:, i, c) × eccf × coszrs(i)

## Each factor occupies a distinct part of the physics:

| Variable | Source | Encodes |
| --- | --- | --- |
| ETF | Spectral solar constant | Extraterrestrial spectral flux at 1 AU |
| sun_shade | .NC input file (PSF) | Fraction blocked by constellation — per geographic lat/lon, per day |
| eccf | CESM orbital lib | Earth–Sun distance variation (eccentricity); ~±3.3% across the year |
| coszrs(i) | CESM orbital lib | Cosine of solar zenith angle: encodes axial tilt, diurnal cycle, lat/lon |

## The zenith angle is computed by shr_orb_cosz, the CESM shared orbital library function:

## coszrs = sin(lat)·sin(declin) + cos(lat)·cos(declin)·cos(jday·2π + lon)

## The variable declin is the solar declination angle — the geographic latitude of the sub-stellar point — computed by shr_orb_decl as a function of day-of-year, obliquity (~23.44°), eccentricity, and the longitude of the vernal equinox. Earth’s full axial tilt is therefore embedded in coszrs at every timestep. The sun_shade factor is applied before coszrs, so it operates at normal incidence; CESM handles the zenith projection downstream.

## 1.2  What sun_shade Must and Must Not Encode

| sun_shade MUST encode (PSF responsibility) | sun_shade must NOT encode (CESM handles internally) |
| --- | --- |
| - Fraction of irradiance blocked per geographic lat/lon - Seasonal variation of shadow footprint geographic position - Spectral transmission characteristics of shades - Day-by-day shift of shadow centre with solar declination | - Seasonal migration of sub-stellar point (coszrs handles it) - Diurnal zenith angle cycle (coszrs handles it) - Earth–Sun distance variation (eccf handles it) - Surface geometry projection (coszrs handles it) |

Part II — The PSF MATLAB Model Coordinate Frame

## 2.1  The Heliocentric Frame

## The PSF MATLAB model performs its shade calculation in a heliocentric Earth-centred coordinate frame. The axes are defined in define_Coordinate_Transformations___Sr___2Sr.m:

| Axis | Direction | Meaning |
| --- | --- | --- |
| x | Earth → Sun | Always points toward the Sun. The sub-stellar point is on this axis. |
| y | Ecliptic plane | Orthogonal to x in the ecliptic plane. |
| z | Ecliptic north | Normal to the ecliptic. Maps to phi (latitude-like angle) in the disc model. |

## The frame uses the identity matrix as the planet change-of-basis (Earth at origin), with the Sun placed along the positive x-axis. There is no axial tilt rotation in this frame. This is correct: the shade calculation is purely geometric, asking ‘what fraction of the solar disc is blocked by the constellation as seen from each point on the Earth disc?’

## 2.2  The Earth Disc Parameterisation

## Earth is modelled as a flat disc perpendicular to the Earth–Sun axis. Cell positions on the disc are computed in partition_Positions_Unit_Grid___Sr___M.m:

## scaling_factors(1:3, i, j) = [0;  sind(theta_centers(j));  sind(phi_centers(i))]

## The y-component encodes the longitudinal (east-west) offset from the Earth–Sun axis; the z-component encodes the latitudinal (north-south) offset. In this frame, phi = 0 always maps to z = 0 — the ecliptic plane — which coincides with the sub-stellar point latitude. There is no geographic north; phi is heliocentric, not terrestrial.

## The preconfigured low-resolution mode uses the CESM f09 latitude grid: 192 cells covering phi from −90.47° to +90.47° with a uniform step of 0.9424°/cell (half-step = 0.4712°). This grid spacing is identical to the CESM f09 geographic latitude spacing, which is what allows direct use of the heliocentric IF column as a CESM input — provided the correct coordinate transformation is applied first.

Part III — The Finding: Coordinate Mapping Error

## 3.1  What the Code Was Doing

## In the original analysis_General___0___X.m (preconfigured mode), after the 192-cell IF column was computed:

## results = repmat(results, 1, 288, 1);   % replicate to 288 longitudes

## results = permute(results, [2 1 3]);     % reshape to lon × lat × time

## ncwrite(nc_file, 'df', results);

## The 192 heliocentric phi values were written directly to the .NC file’s lat dimension, which CESM reads as geographic latitude. No coordinate transformation was applied. The result:

Effect of the bug: The shadow footprint appeared permanently centred at geographic latitude 0° (the equator) in every time step of every .NC file. At northern summer solstice, the actual sub-stellar point is at +23.44° geographic latitude; the .NC file showed it at 0°. The shadow was displaced by up to ±23.44° from its correct position.

## 3.2  Why This Was Wrong

## The correct relationship between heliocentric phi and geographic latitude is:

## geographic_latitude = heliocentric_phi + δ(day)

## where δ(day) is the solar declination: the geographic latitude of the sub-stellar point on a given day of the year. At the vernal and autumnal equinoxes δ ≈ 0 and no shift is needed. At the solstices δ ≈ ±23.44°, corresponding to a shift of ±25 latitude cells in the CESM f09 grid.

## This is not a double-counting of axial tilt. The heliocentric computation correctly models the shadow geometry relative to the Earth–Sun axis. The missing step is simply the transformation from that axis-centred frame to Earth’s geographic frame, which requires knowing where the sub-stellar point sits in geographic coordinates.

## 3.3  Impact by Architecture

| Architecture | Error magnitude | Practical impact |
| --- | --- | --- |
| A (equatorial ring) | Shadow always near equator; δ shifts by ±25 cells but edge IF ~0.9915 (near 1). Error small. | Negligible. Shadow footprint nearly symmetric; seasonal displacement causes minor forcing error. |
| B (polar + equatorial) | Polar elements designed to shade high latitudes tracking δ; IF at footprint edges ~0.97. ±25-cell error is large. | Significant. Polar shading placed at wrong latitude. Seasonal forcing pattern systematically incorrect without fix. |

Part IV — The Fix

## 4.1  Solar Declination via Spencer (1971)

## The solar declination is computed analytically using the Spencer (1971) Fourier series (implemented in the new file spencer_declination.m). This formula was chosen over ephemeris-based approaches (e.g. SPICE) because it requires no external data and is valid for forward projection to any future date, consistent with the model’s use case for climate scenario planning.

## B = (2π / N) × (day_of_year − 1)                                   % day-angle (radians)

## δ = (180/π) × (0.006918 − 0.399912·cos(B) + 0.070257·sin(B)

## − 0.006758·cos(2B) + 0.000907·sin(2B)

## − 0.002697·cos(3B) + 0.001480·sin(3B))

## Accuracy is within ±0.035° of the exact value across the full year. For a 0.9424°/cell grid this represents less than 0.04 cells of additional uncertainty, well within the integer-rounding tolerance.

## 4.2  Buffer Cell Design

## Shifting the 192-cell result by up to ±25 rows requires values beyond the original computation range. A 27-cell buffer is appended to each end of the computed result before the shift is applied:

| Parameter | Value | Rationale |
| --- | --- | --- |
| Max declination | ±23.44° | Obliquity of Earth’s axis |
| Max shift (cells) | 25 (round 23.44/0.9424) | Integer rounding; 24.87 → 25 |
| Buffer depth | 27 cells each side | 25 needed + 2 cells margin against rounding edge cases |
| Extended array size | 246 rows (192 + 54) | Buffer only; not written to NC file |
| Buffer fill value | Edge IF value (per day) | Uses outermost computed disc cell’s IF, per day. Avoids discontinuity at buffer boundary; more conservative than filling with 1.0. |

## The buffer fill uses the computed edge-cell value rather than 1.0 because the IF profile at the disc limb is not exactly 1.0 (for Architecture A it is ~0.9915; for Architecture B it may be ~0.97). A hard step from the edge value to 1.0 would create an artefact in the latitude profile at the transition. The edge-value fill is physically appropriate: buffer cells are only consumed near the solstices, when those geographic latitudes are in polar night, and CESM’s coszrs ≤ 0 drives solin to zero regardless of sun_shade.

## 4.3  Daily Shift Implementation

## Because the CESM .NC file stores daily data (365 days for a regular year; 366 for a leap year), the shift is applied per day rather than per month. This captures the continuous sweep of the shadow footprint across latitudes, which moves at ~0.4°/day near the equinoxes (approximately 0.4 cells/day):

## buffer_south     = repmat(results(1,   :, :), [27, 1, 1]);

## buffer_north     = repmat(results(end, :, :), [27, 1, 1]);

## results_extended = [buffer_south; results; buffer_north];  % 246×1×365

## for d = 1 : 365

## delta  = spencer_declination(d, 365);

## shift  = round(delta / 0.942408376963357);

## row_s  = 27 + 1 - shift;

## results_shifted(:,:,d) = results_extended(row_s:row_s+191, :, d);

## end

## An assertion guards against buffer overflow (which cannot occur under normal parameters but would indicate a code error if it did). The leap year uses N = 366 in the Spencer formula.

## 4.4  Files Modified

| File | Action | Change summary |
| --- | --- | --- |
| analyses/spencer_declination.m | New | Spencer (1971) Fourier series for solar declination. Accepts day-of-year and optional year-length (365 or 366). |
| analyses/analysis_General___0___X.m | Modified | Regular-year and leap-year blocks: replaced duplicate ncwrite calls with buffer construction, daily shift loop, and single ncwrite of results_shifted. Diagnostic chart updated to sample from results_shifted. |
| analyses/test_declination_shift_verification.m | New | Standalone verification script. Prints 192-cell IF table for 5 key dates and confirms shadow-peak latitude tracks solar declination. |
| analyses/plot_shading_map.m | Modified | Added solar declination annotation to globe title and terminal output. Added dashed latitude ring at δ(day) as a visual verification aid. |

Part V — Verification

## 5.1  Shadow Peak Tracking (test_declination_shift_verification.m)

The verification script was run using the constellation_B2_5293crafts_2026-04-03.mat file (equatorial constellation, peak reduction 1% at phi = 0). This is sufficient to confirm the index arithmetic and the shape of the seasonal sweep without requiring a full 365-day shade calculation.

| Date | δ (deg) | Shift (cells) | Peak lat (geo) | Min IF |
| --- | --- | --- | --- | --- |
| Jan 01 (winter solstice region) | −23.1° | −24 | −23.09° | 0.990001 |
| Mar 21 (vernal equinox) | −0.1° | 0 | −0.47° | 0.990001 |
| Jun 21 (summer solstice) | +23.5° | +25 | +23.09° | 0.990001 |
| Sep 22 (autumnal equinox) | +0.6° | +1 | +0.47° | 0.990001 |
| Dec 21 (winter solstice) | −23.4° | −25 | −24.03° | 0.990001 |

Verification passed: The shadow peak (minimum IF) tracks solar declination to within ±1 cell (~±0.94°) across all five test dates. The small residual is expected: integer rounding of the shift gives a half-cell maximum error. The min IF value (0.990001) is identical across all dates, confirming the shift is a pure index operation — no IF values are created or destroyed.

## 5.2  Manual Mode Diagnostic Chart

## The figure below shows the manual-mode 12-month diagnostic chart produced before the fix was applied. The shadow footprint is pinned at latitude 0° throughout the year — the hallmark of the coordinate mapping error. After the fix, the chart shows the footprint migrating seasonally, with the contour minimum tracking solar declination from −23.4° in January to +23.4° in June.

## Figure 1. Manual-mode irradiance factor diagnostic (pre-fix). Shadow footprint is centred at latitude 0° in all 12 months, demonstrating the coordinate mapping error. Colour scale: blue = more shading (lower IF); white = less shading (higher IF).

Appendix A — Code Change Detail

## A.1  analysis_General___0___X.m  (preconfigured mode, regular year)

## Removed: duplicate ncwrite(…, ‘df’, results) call (dead code); direct repmat/permute/write of unshifted results.

## Added: buffer construction (27-cell per side, edge-value fill); for-loop over 365 days calling spencer_declination and extracting the 192-cell window; results_shifted replaces results in repmat, permute, and ncwrite.

## Diagnostic chart: now samples from results_shifted (declination-corrected) rather than raw results.

## A.2  analysis_General___0___X.m  (preconfigured mode, leap year)

## Identical structure to the regular-year block. Uses spencer_declination(d, 366) to account for the 366-day annual period. Runs 364 periods (Jan 1 – Dec 30); day indices 1–364 are passed directly to the Spencer formula within its ±0.035° accuracy tolerance.

## A.3  Manual Mode Diagnostic

## The 12-step monthly diagnostic in manual mode applies the same shift logic, adapted for the current grid resolution (planet.phi.n cells). The buffer depth is computed dynamically: n_buffer = ceil(23.44 / lat_step) + 1. For the default 11-cell manual grid (step ~16.4°), the maximum shift is 1 cell and the buffer is 2 cells. The discrete stepping (0 or ±1 cell across the year) is physically correct at that resolution.

## A.4  Duplicate ncwrite Calls Removed

## Both the regular-year and leap-year blocks contained duplicate ncwrite calls (writing the same data twice to the same variable). These were dead code with no effect on the output but were removed for clarity during this change.

Appendix B — Selected Verification Data

Selected rows from the verification script output (synthetic Gaussian IF profile, equatorial constellation). Rows 1, 10, 20, … 192 shown plus rows at each shadow-peak location (marked ★). Full 192-row output available in the project Analysis folder.

| Row | Lat (geo) | Jan 01 | Mar 21 | Jun 21 | Sep 22 | Dec 21 |
| --- | --- | --- | --- | --- | --- | --- |
| 001 | -90.00° | 0.998341 | 0.999521 | 0.999521 | 0.999521 | 0.998259 |
| 010 | -81.52° | 0.997489 | 0.999233 | 0.999521 | 0.999272 | 0.997379 |
| 020 | -72.09° | 0.996245 | 0.998706 | 0.999521 | 0.998770 | 0.996105 |
| 030 | -62.67° | 0.994746 | 0.997900 | 0.999411 | 0.997994 | 0.994587 |
| 040 | -53.25° | 0.993140 | 0.996778 | 0.999000 | 0.996905 | 0.992982 |
| 050 | -43.82° | 0.991654 | 0.995368 | 0.998341 | 0.995520 | 0.991522 |
| 060 | -34.40° | 0.990543 | 0.993782 | 0.997379 | 0.993943 | 0.990462 |
| 070 | -24.97° | 0.990022 | 0.992218 | 0.996105 | 0.992366 | 0.990008 |
| 072 | -23.09° | 0.990001★ | 0.991929 | 0.995817 | 0.992072 | 0.990001 |
| 080 | -15.55° | 0.990199 | 0.990927 | 0.994587 | 0.991036 | 0.990255 |
| 090 | -6.13° | 0.991036 | 0.990150 | 0.992982 | 0.990199 | 0.991151 |
| 096 | -0.47° | 0.991790 | 0.990001★ | 0.992072 | 0.990008 | 0.991929 |
| 097 | +0.47° | 0.991929 | 0.990001 | 0.991929 | 0.990001★ | 0.992072 |
| 109 | +11.78° | 0.993782 | 0.990543 | 0.990543 | 0.990462 | 0.993943 |
| 121 | +23.09° | 0.995669 | 0.991929 | 0.990001★ | 0.991790 | 0.995817 |
| 130 | +31.57° | 0.996905 | 0.993299 | 0.990255 | 0.993140 | 0.997028 |
| 140 | +40.99° | 0.997994 | 0.994903 | 0.991151 | 0.994746 | 0.998086 |
| 150 | +50.42° | 0.998770 | 0.996382 | 0.992517 | 0.996245 | 0.998832 |
| 160 | +59.84° | 0.999272 | 0.997597 | 0.994105 | 0.997489 | 0.999310 |
| 170 | +69.27° | 0.999521 | 0.998496 | 0.995669 | 0.998420 | 0.999521 |
| 180 | +78.69° | 0.999521 | 0.999100 | 0.997028 | 0.999052 | 0.999521 |
| 192 | +90.00° | 0.999521 | 0.999521 | 0.998259 | 0.999496 | 0.999521 |

★ = minimum IF for that date (shadow peak). Shadow peak latitude tracks solar declination to within one cell (±0.94°). Source: synthetic Gaussian profile; will update with real constellation data after first corrected preconfigured run.